Changing Degree Temperature Display
Press and hold the recirculation button. Then press the temperature up ("+") button to switch between degrees Celsius & Fahrenheit on the Climate Control Temperature and Instrument Panel Outside Temperature displays. 

Accessing On-Board Diagnostic codes
Press and hold the recirculation button. Then press the manual flow control up arrow. You should see a 1c. Press the temperature up ("+") or down ("-") buttons to select a code number. Then press the recirculation button again. The value should display. Press the temperature up or down button again to display another code. 

NOTE: Air Flow Motor (V 71) and Potentiometer (G 113) are not installed in USA/Canada vehicles. Disregard values displayed for these components. 


Code
Displayed Value

1
System malfunction - displayed as a Diagnostic Trouble Code (DTC), see chart below

2
Digital value of Interior Temperature Sensor, in Headliner (G 86)

3
Digital value of Interior Temperature Sensor, in Instrument Panel (G 56)

4
Digital value of Fresh Air Intake Duct Temperature Sensor (G 89)

5
Digital value of Outside Air (Ambient) Temperature Sensor (G 17), front

6
Digital value of Outside Air (Ambient) Temperature Sensor

7
Digital value of Ambient Temperature Sensor At Fresh Air Blower (G 109)

8
Digital value of Temperature Regulator Flap Motor Potentiometer (G 92)

9
Delta value of Temperature Regulator Flap

10
Non-corrected specified value of Temperature Regulator Flap

11
Digital value of Central Flap Motor Potentiometer (G 112)

12
Specified value of Central Flap

13
Digital value of Footwell/Defroster Flap Motor Potentiometer (G 114)

14
Specified value of Footwell/Defroster Flap

15
Digital value of Air Flow Flap Motor Potentiometer (G 113)

16
Specified value of Air Flow Flap

17
Vehicle Speed (km/h)

18
Actual Air Blower voltage (Volts)

19
Specified Fresh Air Blower voltage (Volts)

20
A/C Compressor (A/C Clutch) voltage (Volts)

21
Number of low voltage occurrences, non-transient

22
Cycle condition of A/C Refrigerant High Pressure Switch (F 118)

23
Cyclings of the A/C Refrigerant High Pressure Switch (F 118)

24
Cyclings of the switches, absolute non-fluctuating

25
Analog/Digital value, Kick-Down Switch

26
Analog/Digital value, Engine Coolant Temperature (ECT) Warning Light

27
Coding value

28
Engine Speed (RPM)

29
A/C Compressor speed in rpm (Equals Engine Speed x 1.28)

30
Software version

31
Display check (all segments of A/C Control Head display light up)

32
Potentiometer malfunction counter, Temperature Regulator Flap

33
Potentiometer malfunction counter, Central Flap

34
Potentiometer malfunction counter, Footwell/Defroster Flap

35
Potentiometer malfunction counter, Air Flow Map

36
Feedback value, cold end-stop, Temperature Regulator Flap Motor Potentiometer (G 92)

37
Feedback value, hot end-stop, Temperature Regulator Flap Motor Potentiometer (G 92), max. stop

38
Feedback value, cold end-stop, Central Flap Motor Potentiometer (G 112)

39
Feedback value, hot end-stop, Central Flap Motor Potentiometer (G 112)

40
Feedback value, cold end-stop, Footwell/Defroster Flap Motor Potentiometer (G114)

41
Feedback value, hot end-stop, Footwell/Defroster Flap Motor Potentiometer (G114)

42
Feedback value, cold end-stop, Air Flow Map Motor Potentiometer (G 113)

43
Feedback value, hot end-stop, Air Flow Map Motor Potentiometer (G 113)

44
Vehicle operation cycle counter

45
Calculated interior temperature (internal software, in digits)

46
Outside (ambient) temperature, filtered, for regulation (internal software)

47
Outside (ambient) temperature, unfiltered, (internal software, in deg C)

48
Outside (ambient) temperature, unfiltered, (in digits)

49
Malfunction counter for speedometer (vehicle speed) signal

50
Standing time (in minutes)

51
Engine Coolant Temperature (ECT) in deg C

52
Graphics channel 1 - A/C compressor switch-off conditions are identified by illuminated segments of the ?88.8? display. See chart below.

53
Graphics channel 2 - Climate system electrical outputs are identified by illuminated segments of the ?88.8? display. See chart below.

54
Control characteristics

55
Outside (ambient) temperature, in deg C or deg F depending on setting on A/C control head

56
Temperature in deg C, from Interior Temperature Sensor, in Headliner (G 86)

57
Temperature in deg C, from Interior Temperature Sensor, in Instrument Panel (G 56)

58
Temperature in deg C, from Fresh Air Intake Duct Temperature Sensor (G 89)

59
Temperature in deg C, from Outside Air (Ambient) Temperature Sensor (G 17), front

60
Temperature in deg C, from Ambient Temperature Sensor At Fresh Air Blower (G 109)

61
Software version (latest)